BEST PRACTICE TO TRANSPORT

Purpose:

Maersk is committed to ensuring the overall safety of maritime transportation. Therefore, we have created this easy reference document as a step-by-step guide to book certain commodities, which is intended to support the safety of our crew at sea and shoreside, as well as safety of cargo, environment, vessels, and facilities. This Best Practice gives you the needed guidance on correct stuffing.

目的：

Maerskは海上輸送の安全確保に全力を尽くしています。 そのため、海上および陸上勤務の乗組員の安全、貨物、環境、船舶、施設の安全を確保することを目的として、特定の貨物を予約するためのわかりやすい参考資料を作成しました。 このガイドでは、正しい積み込みに関する必要な指針を示しています。

Commodity:

Steel Coils > 2.0 mt

品目：

Steel Coils > 単重2.0トン以上

Maersk.com commodity description:

Steel & other metal Coils, Sheets, Pipes, Bars, Tube articles with per unit weight more than 2 tons

Maersk.comの品目：

Steel & other metal Coils, Sheets, Pipes, Bars, Tube articles with per unit weight more than 2 tons

Booking via other channels

002317 Steel & other metal Coils, Sheets, Pipes, Bars, Tube articles with per unit weight more than 2 tons

他のサイトからのブッキング

002317 Steel & other metal Coils, Sheets, Pipes, Bars, Tube articles with per unit weight more than 2 tons

Description and Definition:

Steel Coils falls under the category of High Density commodity, as the weight is relatively high compared to cargo volume. This condition requires special handling and stuffing.

Steel Coils can be stuffed in special cradles, special stuffing applications (e.g accepted Styrofoam products) or on-site constructed securing arrangement.

We categorize stuffing of coils as follows: eye-to-sky, eye-to-side or eye-to-rear.

説明と定義：

Steel Coilsは、容積に対して重量が比較的重いため、高密度貨物に分類され、 特別な取り扱いと梱包が必要です。

Steel Coilsは、専用のクレードル、専用の詰め物（例えば、認められた発泡スチロール製品）または現場で組み立てた固定装置に入れることも可能です。

コイルの詰め方はeye-to-sky, eye-to-side または eye-to-rearの3種類に分類されます。

Synonyms:

Coiled metal sheets, wire coils, electric steel sheets, tin- or galvanized plates

類似品:

Coiled metal sheets, wire coils, electric steel sheets, tin- or galvanized plates

Risk:

Main risk is steel coils breaking loose from its securing arrangement and exit the unit through walls or flooring.

Condensation and damage to cargo is also an associated risk.

主なリスク：

Steel Coilsが固定具から外れて、壁や床を突き破ってコンテナの外に飛び出すリスクがあります。

また結露や貨物の損傷も関連リスクです。

Stuffing Q&A:

Stuffingに関するQ&A：

Q1. What Transportation Unit can be used?

A1. We accept this cargo in 20” standard, 40” standard and HC, and Flatracks.

SOC may be used if accepted as per MSK internal procedures.

Q1. どの輸送用コンテナを使用できますか？

A1. 20ft Dry 、40ft DryとHigh Cube、40 Flat Rackで輸送可能です。

SOCは、MSK社内手続きに従って承認されれば使用可能です。

Q2. What Transportation Unit cannot be used?

A2. Non-operating reefers, Refrigerated units and 45” standard dry or HC.

Q2. 輸送コンテナのうち、使用できないものはありますか？

A2. Non-operating reeferまたはReefer、45ft DryとHigh Cubeです。

Q3. How are coils safely secured for ocean voyage?

A3. Cargo shall be stuffed in accordance with CTU code. This includes securing for tipping and sliding in

longitudinal and transverse direction. Forces towards side- and end-wall shall be distributed evenly

throughout the full length/width.

Forces towards door-end shall be arrested by a bulkhead/barrier anchored in the corner posts.

Coils shall be lashed to avoid crippling during the voyage.

Additionally, point load per running mtr or m2 cannot be exceeded.

Q3. 海上輸送上でコイルを安全に固定するには？

A3. 貨物はCTU codeに従って積み込む必要があります。 これには、縦方向および横方向の転倒防止策と滑り止め対策が含まれます。 側面および端壁にかかる力は、長さおよび幅全体に均等に分散させるものとします。扉端にかかる力は、コーナーポストに固定された隔壁/バリアによって受け止めるものとします。

コイルは、航海中に破損しないようしっかりと固定しなければいけません。

また、1 メートルまたは 1 平方メートルあたりの点荷重は、規定値を超えてはいけません。

Q4. How to calculate allowable point load?

A4 This is calculated by using unit payload divided by internal length or loadable area.

Example of 8,0 tn coil in 20” standard dry: Payload / internal length = 28300kg/5,90mtr = 4.796 kg/mtr.

Calculation of bedding: Coil weight / allowable point load = 8.000kg/4.796 kg/mtr = 1,67mtr

Weight must be distributed on bedding being minimum 1,67mtr

Q4. 許容点荷重の計算方法は？

A4. こちらは、コンテナのpayloadをコンテナ内側の長さまたは積載可能面積で割って算出します。

20ft Dryに積載された8,000kgのコイルの例：コンテナのpayload/コンテナ内側の長さ=28,300kg/5.90メートル=4,796kg/メートル。

ベッドの計算：コイル重量/許容点荷重=8,000kg/4,796kg/メートル=1.67メートル

重量は、ベッドに最低1.67メートルにわたって均等に分散させる必要があります。

Q5. Can dunnage bags be used for securing?

A5 Maersk have decided NOT to accept dunnage bags due to the variety of application and quality of

dunnage bags. An exemption of this restriction might be applied for.

Q5. ダンネージバッグは固定用として使用できますか？

A5ダンネージバッグの用途や品質にはばらつきがあるため、Maerskではダンネージバッグの使用を認めておりません。

Mandatory Documentation:

After booking, customers will receive a request for documentation that shows the following:

• Coil(s) weight and dimension.

• Stuffing methodology.

• Coil weight is distributed on sufficient bedding to avoid exceeding point load.

• Sufficient securing for longitudinal and transverse movements.

• Coil(s) is prevented from tipping.

• Coil(s) is lifted from floorboards.

• Forces towards door end is arrested in corner post by a bulkhead.

必要事項：

ブッキング後、お客様には以下の内容が記載された書類（Maerskの書式）をご提出いただきます。

• コイルの重量と寸法（メールでお知らせください）

• 積み込み方法

• コイルの重量が一点荷重を超えないように、適正な敷き詰め材の上に分散すること

• 縦方向および横方向の動きを阻止するよう十分に固定すること

• コイルが転倒しないよう固定すること

• コイルがコンテナの床に触れていないこと

• コンテナドアへの動きを阻止するために、バルクヘッドを取り付けること

Links to Additional Information & Industry Standards:

参考情報リンク:

Code of Practice for Packing of Cargo Transport Units (CTU Code)

Informative Material Related to the IMO/ILO/UNECE Code Of Practice For Packing Of Cargo Transport Units (CTU Code)

CINS / TT Club Transport of Coiled Materials in Containers

Latest update – 25-07-2024