DANGEROUS CARGO – QUICK GUIDE OF BOOKING PROCESS - Cargo acceptance: In general, Maersk Line accepts dangerous cargo according to IMDG (International Maritime Dangerous Goods) regulations. The acceptance however subjects to local country port acceptance (including load, transit, transshipment, last discharge port) and vessel operator. Dangerous booking instruction must include below mandatory information: 2.1 Commodity description, 2.2 IMO class, UN No. 2.3 Packing type / code. 2.4 Number of packages. 2.5 Gross weight, Net weight. 2.6 Emergency contact number / contact name. 2.7 Additional details such as flashpoint, boiling point, marine pollutant… may be requested in some instances. * Attached template can be used as reference booking instruction format - Booking confirmation turn time: DG booking is released within 01 – 03 working days provided by sufficient information submission from customer and approval(s) obtained from responsible desks. - Before loading: - Hazardous cargo certificate - Multimodal Dangerous Goods Form (MDGF) submission is mandatory. This form must be sent with full details (items 15 to 22 (container number, seal number, name of your company…), sign and stamp in both left and right sides) with declarant’s signature, company’s stamp and Material Safety Data Sheet (MSDS) by 48 working hours before cut off time to vn.export@maersk.com via a new, fresh email. Maeersk is not responsible for any rolling/issues happening due to late DGD or DGD mismatched with booking confirmation. - DG cargo mis-declaration: Customers are fully responsible for their correct cargo declaration. Any discrepancy / mismatch found for their dangerous cargo will be charged mis-declaration fee of 15,000 USD per container. Charge applicable: please refer to the link HERE. Other Procedures: For smooth loading of cargoes, customer needs to make further arrangements including but not limited to: + Label 4 sides of the DG cargo container with proper DG labels as IMDG regulations. + Declare the DG details with Terminal when dropping off full containers at terminal. + Stuff cargo or drop off full container as per terminal’s acceptance (Cat Lai terminal: within 24 hours before ETA) - Special requirement of Gas lighter shipment (IMO 2.1, UN 1057): + In order to provide safe journey, any amount of UN 1057 must be measured for presence of gas within 24 hours prior to loading to any vessel at any load or transshipment port, and the certification is presented to the master. + Loading criteria is 0% LEL test (gas survey certificate) measured by digital or analogue catalytic combustion principle for detection. (“LEL Test” – is a test to assess if there is any flammable gas in the container as a percentage of Lower Explosive Limit). + The measurement must be performed by industrial hygienist or another qualified person. Equipment used for measurement must be a gas detector of relevant type. + Any container found with any presence of flammable gas must not be loaded onto any vessel and will follow the process for gas leakage containers. + For loading ports in Vietnam: customer will send the LEL test as mentioned above to Maersk Line Vietnam (email vn.export@maersk.com and SGNOPSGEN@maersk.com) before the CY cut-off time. APPENDIX – Terminology International Maritime Organization (IMO) regulates various aspects of hazardous cargo shipment from classification and identification of each commodity, packing and labelling, stowage and segregation to emergency handling procedures as per the IMDG Code. Shipper shall be expected to have full knowledge about these regulations and specifications of their cargo. - MSDS is Material Safety Data Sheet and should provide: + Laboratory information about the product + Physical and Chemical characteristics about the product + Safety and emergency and disposal handling procedures + Stowage & Transport Information if the product is IMO regulated etc - Flashpoint – The lowest temperature at which a flammable liquid forms an ignitable mixture with air. This will indicate that a container must be kept at a temperature below the flashpoint for safety sake. - Marine Pollutant – Substances that could be toxic to aquatic life. - EMS - EMS is the Emergency schedule, Emergency procedure for ships carrying dangerous goods. It is used in the event of an incident involving dangerous goods. - IMO Class – A classification from 1 – 9 provided by the IMO. Sub risks classifications may apply. - Packing Group – Packing groups represent the degree of hazard. Packing group I is the most dangerous and III the least dangerous. - Proper Shipping Name - Proper Shipping Name is the name of cargo as described in the IMDG Code. Same commodity can be known or named differently in different places. In order to avoid confusion, and speak the same language when shipping hazardous cargo, only the Proper Shipping Name of cargo must be used. - Technical Name - Generic or "Not Otherwise Specified" (N.O.S.) entries must be supplemented with their technical or chemical group names. - UN Number - UN Number is the four digit number assigned to each dangerous goods by the UN (United Nations). With Regards, Maersk